This safety alert has been prepared by the RSTA. Completed forms for adding to the RSTA website should be submitted to Clive Mitchell clive@rsta-uk.org For further information regarding this safety alert please email Clive or phone 07557529410

Safety Alert

Hydraulic Injection Injury

PURPOSE

The purpose of this safety alert is to raise awareness of the risk from hydraulic injection injuries (HII).

BACKGROUND

An incident occurred earlier this year during the maintenance of the tracks by a fitter on another companies piling rig. During tensioning of the track, a release of grease under high pressure struck the fitter. This tragic incident later resulted in the death of the fitter.

HSE INVESTIGATION FINDINGS

Whilst  the  fitter  was  using  a  hand  powered  grease  gun  to  tension  the  track  of  a  piling  rig,  the  nipple connecting the grease gun to the track mechanism detached. This resulted in the grease (under significant stored energy) to release, injecting the fitter.

Initial findings from a HSE investigation have concluded that anyone involved in the maintenance of mobile or static plant where greasing occurs on high pressure systems are

Alert No 01/2015

This safety alert has been prepared by the RSTA. Completed forms for adding to the RSTA website should be submitted to Clive Mitchell clive@rsta-uk.org For further information regarding this safety alert please email Clive or phone 07557529410

Safety Alert

potentially at risk of a HII.

CAUSATION

HII’s often result  from a failure of a component in the mechanical system. This can result in the equipment

remaining in use and pressurised whilst leaking fluid.

A  HII  occurs,  when  the  outer  layer  of  skin  is  broken  by  a  jet  of  fluid  under  pressure.  Although  serious

reported  instances  of  hydraulic  injection  have  occurred  at  pressures  over  100  bar  (1450  psi),  anecdotal

evidence suggests a HII may occur at pressures as low as 7 bar (101.5psi).

A person may come in to contact with a fluid jet while using equipment or when carrying out inspection and

or maintenance prompted by a reduction in performance of the equipment.

This can cause harm in two ways:

- The mechanical pressurised penetration and subsequent trauma to the surrounding and underlying tissue.

- The toxicity of the fluid injected into the injured party.

Alert No 01/2015

Alert No 01/2015

This safety alert has been prepared by the RSTA. Completed forms for adding to the RSTA website should be submitted to Clive Mitchell clive@rsta-uk.org For further information regarding this safety alert please email Clive or phone 07557529410

Safety Alert

The severity of the injury varies according to the following:

- Pressure.

- Proximity

- Jet size.

PHYSICAL EFFECTS OF A HII

- Initial impact from a HII may result in severe bruising, tenderness and swelling of the affected area.

- After four – six hours the victim may experience intense throbbing pain that is unresponsive to pain medication.

- This can then lead to amputation of affected parts and be fatal.

IMMEDIATE ACTIONS FOLLOWING A HII

- Following any HII, prompt action is essential to save the injured persons limb, or even life.

- Staff should receive immediate professional medical treatment.

ADDITONAL PRECAUTIONS / PREVENTATIVE MEASURES

- Ensure  staff  are  wearing  suitable  personal  protective  equipment  as  per  your  risk  assessment findings when conducting maintenance on plant and equipment.

- Employees should  give consideration to general greasing

This safety alert has been prepared by the RSTA. Completed forms for adding to the RSTA website should be submitted to Clive Mitchell clive@rsta-uk.org For further information regarding this safety alert please email Clive or phone 07557529410

Safety Alert

of other parts of plant equipment such  as

greasing the hydraulic rams as these MAY also be pressurised.

- All  maintenance  on  a  pressurised  system  should  be  in  accordance  with  the  manufacturer’s

specifications.

- Where replacement or maintenance takes place on a pressurised system, fittings should be matched

and be compatible as per the original equipment manufacturers specification.

- Verification of such replacement should be regarded as a pressure  test and appropriate precautions

taken.

NOTE: This alert relates to the initial HSE preliminary investigation findings and the subsequent HSE alert that

has been  issued.  Any  further  recommendations  from  the  HSE  will  be  communicated  at  a  later  date  (if required).

This alert is to be communicated to all personnel or subcontractors who are working on a BSHEL  site  or  within  a  BSHEL  workshop,  that  are  involved  in  the  maintenance  and greasing” of plant and equipment.

Alert No 01/2015