Traffic Management

Description of incident

A near hit/potential incident occurred

when footway resurfacing works

were carried without adequate or

Chapter 8 compliant traffic and

pedestrian management.

The site had a speed limit of 60mph

with no pedestrian crossing point or

an alternative footpath.

Injury

No injuries occurred, however this could have easily resulted in a serious incident with potential damage to the operatives or a member of the public.

- Traffic management or Pedestrian Management

Alert No 09/2015

This safety alert has been prepared by the RSTA. Completed forms for adding to the RSTA website should be submitted to Clive Mitchell clive@rsta-uk.org For further information regarding this safety alert please email Clive or phone 07557529410

Safety Alert

Alert No 09/2015

This safety alert has been prepared by the RSTA. Completed forms for adding to the RSTA website should be submitted to Clive Mitchell clive@rsta-uk.org For further information regarding this safety alert please email Clive or phone 07557529410

Safety Alert

Why did this happen?

- The surfacing crew arrived on site without consulting the paperwork

- The crew completed the works even though they were aware of the risks of setting up on a 60mph site with no Traffic management or Pedestrian Management

- The survey was not competed in conjunction with the Chapter 8 red book

In addition the following factors contributed to the near hit:

- The survey was not reviewed during the programming stage prior to the gang commencing work

- The site survey template is inadequate for traffic management exceeding a standard Give and Take layout

- The Surveyor did not hold 12d T7 training

Actions

- No works must be started where any safety risks or concerns are present – down tools and call Report It immediately!

Alert No 09/2015

This safety alert has been prepared by the RSTA. Completed forms for adding to the RSTA website should be submitted to Clive Mitchell clive@rsta-uk.org For further information regarding this safety alert please email Clive or phone 07557529410

Safety Alert

- All surveys must be reviewed at

quotation stage to ensure they

are compliant with Chapter 8 by

a competent and qualified person

- 12d T7 training is to be carried

out for all staff who will be

carrying out surveys, reviewing

surveys or putting operatives to

work on site

- A new traffic management

assessment form is to be

adopted from the ‘National

Highways Sector Schemes’ on

sites where Traffic Management

over and above ‘give and take’ is

required.