Alert No 10/2015

This safety alert has been prepared by the RSTA. Completed forms for adding to the RSTA website should be submitted to Clive Mitchell clive@rsta-uk.org For further information regarding this safety alert please email Clive or phone 07557529410

Safety Alert

Use of Stihl Saws and dust suppression

What is the issue?

Stone, rock, sand and clay all contain

high levels of crystalline silica, these

materials are used to make kerb

stones, paving slabs, bricks, tiles and

concrete.

Cutting of these building materials with

an abrasive wheel releases silica in the

form of dust which is very harmful if

breathed in, this is called respirable

silica dust.

Health Effects

Respirable silica dust is harmful and causes a stiffening and scarring of the lungs, called silicosis. This makes the sufferer feel breathless and if inhaled in high concentrations this can lead to the person being classed as disabled due to the

Alert No 10/2015

This safety alert has been prepared by the RSTA. Completed forms for adding to the RSTA website should be submitted to Clive Mitchell clive@rsta-uk.org For further information regarding this safety alert please email Clive or phone 07557529410

Safety Alert

severe breathlessness. The damage is irreversible.

How can we protect ourselves ?

- Dust suppression at source

must be deployed to protect

those in the local area

- The key to dust suppression

is having a constant stream

of water to prevent the build

up of dust AND having a

suppression method which

involves fewer persons being

exposed as possible

- The equipment required is

shown in the picture above

right

- With this equipment in use,

the person operating the

Stihl saw is also required to

wear a face mask with a filter

grade FFP3

Alert No 10/2015

This safety alert has been prepared by the RSTA. Completed forms for adding to the RSTA website should be submitted to Clive Mitchell clive@rsta-uk.org For further information regarding this safety alert please email Clive or phone 07557529410

Safety Alert

Actions

- Only use the suppression equipment pictured above,

- The use of watering cans is not an approved method of dust suppression

- When using the Stihl pressurised water bottle, the flow rate should be approximately 0.5 litre per minute, so the 16 litre tank should last 32 minutes

- PPE to be worn: Safety glasses/goggles, face mask with FFP3 grade filter, gloves, ear defenders, this is in addition to the usual high visibility clothing and safety boots.

- Dust suppression kits not working must be reported to Report It immediately, do not continue cutting without this safe system of work in place, it is there for your protection